Large Non-Residential Establishments in or Surrounded by Large Lot Residential Districts

Certain non-residential uses that are allowed in large lot residential districts have the potential to create negative impacts on large lot residential neighborhoods when the non-residential uses are very significantly larger and more intensive than the typical development in these areas.  This section provides consistent standards for such cases.

- Applicability

The standards of this section shall apply to any institutional, commercial, educational, recreational use, or utility facility or combination thereof where the gross area of all structures on a lot or tract exceed 10,000 square feet, either when such facility is within any Class B zoning district or when the parcel under development abuts property in a Class B district for more than 50% of its boundary.

- Relationship to Other Standards

The provisions of this section are in addition to other requirements of this title, and may be more stringent than other requirements of this title.  Where there is a conflict with other applicable standards, the more stringent standard shall apply.

- Setbacks

Structures over 35 feet in height shall be setback beyond the underlying side or rear setbacks of the district by an additional 20 feet.

- Parking

- Parking lots for 20 to 50 vehicles shall not be located within setbacks equal to those for the adjoining large lot residential district.

- Parking lots for over 50 vehicles, queuing zones, passenger loading zones, and/or delivery zones, shall not be located in the setbacks equal to those for the adjoining large lot residential district and must be buffered by L3 screening landscaping.

- Vehicle storage and fleet vehicle parking are not allowed within setbacks equal to those for the adjoining district.

- Buffers

L3 buffer landscaping shall be provided along any lot boundary with R6, R7, R8, R9 or R10 zoning, except as provided elsewhere in this section.

- Vegetated Open Space

A minimum of 35 percent of the lot area shall remain as planted open area, landscaped area or natural vegetation area, to exclude buildings, driveways, parking areas, sidewalks, etc. unless the decision-making body determines that retention of less than 35 percent provides a development character in keeping with the surrounding neighborhood.

- Architectural Features or Screening for Large Facades

Facades longer than 100 feet that face residential lots or public streets shall have façade modulations at least every 60 feet or eight feet of foundation landscaping.