    - Large Non-Residential Establishments in or Surrounded by Large Lot Residential Districts

Certain non-residential uses that are allowed in large lot residential districts have the potential to create negative impacts on large lot residential neighborhoods when the non-residential uses are significantly larger and more intensive than the typical development in large lot residential districts.   This section provides consistent standards for such uses that meet the applicability threshold below.

      - Applicability

- The standards of this section shall apply to any roominghouse, child care center, neighborhood recreation center, religious assembly, educational facility (elementary, middle, or high school, or instructional services), commercial horticulture, commercial kennel, veterinary clinic, or recreational or vacation camp use, cultural facility, large domestic animal facility (principal use), utility facility, or any combination thereof, where the sum of the gross floor area of all structures on the lot is more than 6,000 square feet or the parking is 10 or more spaces, and the use is on a lot zoned R-6, R-7, R-8, R-9, or R-10, or on a lot with R-6, R-7, R-8, R-9, and/or R-10 residential zoning abutting more than 50 percent of the lot boundary.

      - Relationship to Other Standards

The provisions of this section are in addition to other requirements of this title, and may be more stringent than other requirements of this title.  Where there is a conflict with other applicable standards, the more stringent standard shall apply.

      - Floor Area Ratio

The maximum floor area ratio (FAR) of all buildings on the lot shall be 0.30.

      - Setbacks

        - Structures that are over 20 in height feet shall be setback beyond the underlying side or rear setbacks of the district by one foot per foot of height over 20 feet.

        - Structures that are over 45 feet in height shall have a solar access impact analysis, and the building mass or placement may be altered to ensure that existing solar access to adjacent lots is retained for at least four hours of sunlight on March and September 21.

      - Parking

        - Parking lots for 10 to 20 vehicles shall not be located within setbacks equal to those for the adjoining large lot residential district.

        - Parking lots for over 20 vehicles, queuing zones, passenger loading zones, and/or delivery zones, shall not be located in the setbacks equal to those for the adjoining large lot residential district and must be buffered by L4 screening landscaping.

        - Vehicle storage and fleet vehicle parking are not allowed within setbacks equal to those for the adjoining district.

      - Buffers

- L3 buffer landscaping shall be provided along any lot boundary with R-6, R-7, R-8, R-9, or R-10 zoning, except as provided elsewhere in this section.

      - Vegetated Open Space

- A minimum of 35 percent of the lot area shall remain as planted open area, landscaped area, or natural vegetation area, to exclude buildings, driveways, parking areas, sidewalks, etc., unless the decision-making body determines that retention of less than 35 percent provides a development character in keeping with surrounding neighborhood.

      - Architectural Features or Screening for Large Facades

- Facades longer than 100 feet that face residential lots or public streets shall have façade articulations at least every 50 feet as in section 21.07.120E.5.c.i., or eight feet of foundation landscaping with one unit per linear foot.

- New standard to be located elsewhere:

- Frame-supported, arch-supported, or inflated tension fabric or membrane structures may not exceed 15 feet in height.